Multi-Modal Biometric ATM

Setup & Run Guide

# Prerequisites

- Python 3.11+ installed — check with: python --version

- A webcam

- The project folder on your machine

# Step 1 — Install Dependencies

Open a terminal in the project folder and run:

pip install -r requirements.txt

Windows note: If insightface fails to install, you may need Visual C++ Build Tools from visualstudio.microsoft.com/visual-cpp-build-tools. Run the installer and select "Desktop development with C++".

First run note: InsightFace will automatically download the buffalo_l face model (~300MB) on first use. This needs internet once, then works offline.

# Step 2 — Verify the Setup

Run the self-test to confirm all components are working:

python main.py --test

You should see 5 checks. A fresh install will pass Database and Webcam. Face Encodings and Fingerprint will warn “not found” — that is expected until you enroll a user.

# Step 3 — Enroll a User

This registers a person into the system. Run:

python main.py --enroll

The prompts will walk you through:

- a) User ID

- Enter something like USR001

- b) Full Name

- e.g. John Doe

- c) PIN

- Enter a 4-digit PIN, then confirm it

- d) Face capture

- Choose A for Auto (easier) or M for Manual

- A webcam window opens. Look at the camera. It captures 20 photos automatically.

- Keep your face well-lit, centered, and try a few slight angle variations.

- Press Q to quit early if needed.

- e) Fingerprint image

- You will be asked to paste a file path to a fingerprint image.

- You need a fingerprint image file (JPG/PNG/BMP). For demo/testing, download a sample from the SOCOFing dataset on Kaggle.

- Paste the full file path, e.g. on Windows:  C:\Users\YourName\Downloads\fingerprint.bmp

- After enrollment completes, you will see a success summary.

- Repeat this step for each person you want to register.

# Step 4 — (Hardware Mode Only) Flash the ESP32

Skip this step entirely if using software fingerprint mode (the default).

If you want to use the physical R307S sensor:

- Open esp32/fingerprint_auth/fingerprint_auth.ino in Arduino IDE

- In Arduino IDE: Tools → Manage Libraries → search and install "Adafruit Fingerprint Sensor Library"

- Wire the R307S to the ESP32 (see docs/wiring_diagram.md)

- Select the correct board: Tools → Board → ESP32 Dev Module

- Select the correct port: Tools → Port

- Click Upload

- In config.py, set SOFTWARE_FINGERPRINT = False

Find the ESP32’s COM port:

python main.py --list-ports

If auto-detection does not work, override it:

python main.py --port COM3    (use whichever port shows up)

# Step 5 — Launch the ATM Interface

python main.py

The GUI opens with a live camera feed on the left and the authentication pipeline steps on the right.

# Step 6 — Authenticate

Click the Start Authentication button and follow the pipeline:

## Stage 1 — Face Recognition

- Look at the webcam. The system needs 5 consistent confirmations (up to 12 seconds).

- It will show the identified name and confidence percentage in the UI.

## Stage 2 — Liveness Check

- Blink naturally 3 times. The counter updates live.

- This prevents someone holding up a photo of you.

- You have 10 seconds.

## Stage 3 — Fingerprint

- Software mode: A dialog pops up. Click Browse Image File and select the same fingerprint image you used during enrollment.

- Hardware mode: Place your finger on the R307S sensor when prompted.

## Stage 4 — Identity Fusion

- Automatic. The system cross-checks that the face ID and fingerprint ID belong to the same registered user.

## Stage 5 — PIN Entry

- A numpad dialog appears. Enter your 4-digit PIN using the on-screen buttons.

- You get 3 attempts.

Result: Either Access Granted (green) or Access Denied (red) with the reason shown.

# CLI Quick Reference

| Command | Purpose |
| --- | --- |
| python main.py | Launch ATM UI |
| python main.py --enroll | Register a new user |
| python main.py --test | System health check |
| python main.py --list-ports | Show available COM ports |
| python main.py --list-users | Show all registered users |
| python main.py --encode | Re-generate face encodings from dataset |
| python main.py --simulate | Run with no hardware (auto-passes fingerprint) |
| python main.py --port COM5 | Force a specific serial port |

# Common Issues

| Issue | Fix |
| --- | --- |
| Camera not opening | Try changing CAMERA_INDEX = 0 to 1 or 2 in config.py if you have multiple cameras. |
| Face not recognized | Run python main.py --encode to regenerate encodings, or re-enroll with better lighting. Raise FACE_RECOGNITION_TOLERANCE from 0.40 to 0.45 in config.py to be more lenient. |
| Fingerprint match fails | Use the same (or very similar) image as the one enrolled. Lower FINGERPRINT_MATCH_THRESHOLD from 35 to 20 in config.py if matches are too strict. |
| ESP32 not connecting | Run --list-ports and pass the correct port with --port COMX. Make sure the CH340/CP210x USB driver is installed (search CH340 driver Windows or CP2102 driver Windows). |